INSERT LOGO IF AVAILABLE

Recycled Water Quality Management Plan

(RWQMP)

Name-F11

Recycling Scheme

Version No: X

Date: day/month/year

This document has been prepared by Name-F11. The information contained in this publication is intended for general use of personnel involved in the Name-F11 to assist in the implementation and management of the recycling scheme.

The styles in the template may be altered to suit your organisation’s style. While every effort has been made to include all relevant aspects of a RWQMP in this template, recycling schemes may consider adding other relevant information. Other documents the organisation/scheme have that support the RWQMP can be referred, linked or included as an Appendix to this template.

The template has been prepared by The Department of Health, Western Australia (DOH) based on the Australian Guidelines for Water Recycling Managing Health and Environmental Risk Phase 1 (2006), the Guidelines for the Use of Recycled Water in Western Australia (2010) and other supporting documentation. Users of the template should review the DOHWA web site for any updates Recycled water in WA (health.wa.gov.au)

Development and implementation of RWQMP is regarded as an essential component of assuring recycled water safety and quality.  This RWQMP template is designed to be flexible and suitable for application to all recycled water schemes irrespective of size and complexity. The level of detail in the plan needs to reflect the nature of the risk with those of lower risk being simpler and much shorter than those with higher risk.

All recycling schemes need to complete Appendices A to L. Appendices M to Z may need to be included if relevant for your recycling scheme. For example, if the scheme received the recycled water from another recycled water supplier, Appendix O (Operational and Maintenance Manual of the WWTP) is not required while Appendix M (Recycled Water Supply Agreement) is required.

Document History

| Issue No (version) | Original prepared by | Issued to (description /section revised) | Date | Reviewed by | Revision Date | Approved by | Approval Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
| V1 |  |  |  |  |  |  |  |

This table need to reflect annual reviews and revisions

Foreword

Include:

- Purpose of the RWQMP

The Name-F11 recycled water quality management plan (RWQMP) is a stand-alone document to be used by all parties in the day to day operation and management of the scheme. …

- Scope of the RWQMP

This RWQMP provides an overview of how the 12 elements of the risk management framework of the Australian Guidelines for Water Recycling Managing Health and Environmental Risk has been addressed in the Name-F11

This management plan is a live document in a process of continual development and review.

- Structure of the document

Chapters 1 to 11 of this RWQMP provides the basic information of Name-F11 recycling scheme and the risk management requirements. Additional information is provided in the Appendices

# Contents

Foreword	2

Contents	3

1.	Introduction	11

1.1 Site description	11

1.2 Existing environment	11

1.3 Scheme overview	11

2. Recycled Water Quality Policy	13

2.1 Policy document	13

2.2 Regulatory and formal requirements	13

2.3 Stakeholder engagement	13

3. Roles and Responsibilities	14

3.1. Supplier	14

3.2 Scheme manager	14

3.3 Users	14

3.4 Duty of care holder	14

3.5 List of contact details	15

4. Assessment of the Recycled Water Scheme	16

4.1 Source water (influent)	16

4.2 Water quality objectives	16

4.3 Health risk assessment	17

4.4 Recycled water storage	17

4.4. Reticulation network (distribution system) and application (receiving environment(s)	18

4.5 Systems operation	18

4.6 Prevention of cross connection with potable supplies	18

5. Treatment Train	19

5.1 Treatment description	19

5.2 The HACCP	19

5.3 Operational monitoring and maintenance	19

5.4 System operators competency	19

5.5 Alarms, critical control points and critical limits	19

5.6 Water supply arrangements	20

6. Monitoring Plan	21

6.1 Validation monitoring	21

6.2 Operational monitoring	21

6.3 Verification monitoring	22

7. Employee Training & Public Education	23

7.1 Employee awareness and involvement	23

7.2 Employee training	23

7.3 Training records	23

7.4 Occupational health and safety procedures	23

7.4.1 Recycled water inductions and hand-over procedures	23

7.4.2 Management measures	23

7.5 Public safety	24

7.5.1 Educational material	24

7.5.2 Management measures	24

8 Incident and Emergency Response	25

8.1 Communication	25

8.2 Incident and emergency response protocols.	25

9 Community Involvement and Consultation	26

9.1 Community consultation	26

9.2 Communication	26

10 Documentation and Reporting	27

10.1 Management of documentation and records	27

10.2 Reporting	27

10.2.1 Internal	27

10.2.2 External	27

10.3 Annual report	27

11 Evaluation and Audit	28

11.1 Long-term evaluation of results	28

11.2 Audit	28

11.2.1 External audit	28

11.2.2 Internal audit	28

References	29

Glossary	31

Appendices	34

Appendix A: Scheme Location	35

Appendix B: Drawing of the Recycled Water Scheme from Source to End-use(s)	36

Appendix C: Water Balance	37

Appendix D: Treatment Plant Process Flow Diagrams	38

Appendix E: Health Risk Assessment	39

1. Hazard identification	39

2. Dose response	39

3. Exposure assessment	39

3.1 Routes of exposure	39

4. Risk characterisation	39

5. Risk mitigation	42

Appendix F: Validation and Verification Report	43

1. Introduction	43

2. Sampling methodology	43

3. Results	43

4. Operational/online data	44

5. Influent flow	44

6. Discussion	44

7. Conclusions	44

8. Copy of original results	44

Appendix G: OHS Procedures and Material Safety Data Sheets	45

Appendix H: Sampling Plan	46

1. Safety procedures	46

2. General rules of sampling	46

3. Sample bottle labelling	46

4. Microbiological collection technique	46

5. Chemical collection technique	46

6. Chlorine residual measurement	46

7. Sample preservation and transportation	46

8. Documentation/records	46

9. Site code number(s) and their map location	46

10. List of laboratories	46

Appendix I: Warning Signs	48

Appendix J: Training/ Education	49

1. Operator training manual	49

2. Induction program and hand-over procedures	49

3. Training of sub-contractors	49

4. Training forms	49

5. Educational material	49

Appendix K: Annual Report	50

1. Introduction	50

2. Quality of recycled water	51

2.1 Recycled water quality sampling	51

2.2 Analysis of results	51

3. Emergency and Incident Management	51

4. Complaints	52

5. Audit	52

Appendix L: Incident Management Plan	54

1. Incident classification	54

2. Notification process	54

3. Emergency communications contact list	54

4. Containment and remediation procedures	55

5. Incident cause and investigation procedures	55

Appendix M: Recycled Water Supply Agreement	56

Appendix N: Operational Monitoring and Critical Control Points	57

Appendix O: Operational and Maintenance Manual of the WWTP	59

1. Overview of treatment process	59

2. System Drawings	59

3. Plant Sart-up	59

4. Alarm systems	59

5. Sludge Management	59

6. Equipment	59

7. Materials and chemicals	60

8. Troubleshooting Guide	60

9. Plant safety and personal hygiene	60

10. Plant monitoring	60

11. Sample collection	60

12. Maintenance	60

13 Communications protocol for emergency and contingency situations	60

Appendix P: Operational and Maintenance Manual of the Irrigation System	61

1. Map of the pipeline route from WWTP to irrigation sites	61

2. Irrigation map	61

3. Areas of responsibility	61

4. Maintenance of storage tank, reticulation system	61

5. Irrigation compliance	61

6. Warning Protocol	61

7. Contingency/emergency plan	61

Appendix Q: Validation Report of Individual Treatment Components	62

1. System description	62

2. Validation ranges	62

3. Results	62

Appendix R: Mosquito Management Plan	63

1. Introduction	63

2. Mosquito breeding habitats and human risk	63

3. Risk management measures	63

4. Roles and responsibilities	63

Appendix S: Algae Management Plan	64

1. Introduction	64

2. Risk management measures to prevent algae blooms	64

3. Risk management practices in algae blooms	64

4. Incident Response	64

Appendix T: Land Capability Assessment	65

1. Site Characteristics	65

2. Irrigation assessment requirements	65

3. Irrigation areas	65

4. Soils	65

5. Irrigation application rates (L/m2)	65

Appendix U: Environmental Risk Assessment	66

1. Routes of Exposure and environmental end-points	66

Include:	66

Exposure pathways	66

Receiving environment	66

Environmental end-points (soil, surface water, biota..)	66

2. Environmental risk assessment	66

3. Risk mitigation	66

Appendix V: Environmental Management Plan	67

1. Soils description	67

2. Water resources	67

3. Land management	67

4. Nutrient and irrigation management plan	67

5. Proposed irrigation scheme	67

6. Nutrient application	67

7. Drainage management	67

8. Vegetation in the irrigated area	67

9. Pesticide storage and use	67

10. Sample locations (soil, groundwater, surface water)	67

Appendix W: Odour Management Plan	68

1. Odour assessment	68

2. Odour control measures	68

3. Odour monitoring	68

4. Standard operational procedures	68

5. Contingency plans	68

Appendix X: Noise Management Plan	69

1. Noise assessment	69

2. Noise management	69

3. Noise monitoring	69

4. Standard operational procedures	69

5. Contingency plans	69

Appendix Y: Catchment Characterisation	70

Appendix Z: Audit Report	71

1. Introduction	71

2. Audit Methodology	71

3. Findings	71

4. Discussion	71

5. Recommendations	71

List of Tables

Apply the ‘Table’ style to the table headings in your document and then generate the list (similar to the TOC generation)

Table 1 Summary of the Name-F11 recycling scheme	12

Table 2 Expected/measure water quality parameters of the source water (influent)	16

Table 3 Water quality objectives of the recycled water	17

Table 4 Alarms message, effect on plant and corrective actions	20

List of Figures

Apply the ‘Figure’ style to the figure headings in your document and then generate the list (similar to the TOC generation)

# Introduction

Include a general description of the recycled scheme.

## 1.1 Site description

- Location

Include diagram of scheme location in Appendix A

## 1.2 Existing environment

- Climate

- Land use vegetation

- Topography and flooding

- Proximity to water sources and water table

- Sensitive receiving environments

Include photos or relevant graphs in Appendix A. Include water balance details in Appendix C: Water Balance.

## 1.3 Scheme overview

Include a general description of:

- Source water

- Location of the treatment apparatus  or location where the water is harvested/abstracted

- Proximity to drinking water infrastructure and drinking water sources

- Location and capacity of storage apparatus/ponds

- Treatment apparatus

- Schematic of the pipelines or reticulation network

- End-uses and their location (e.g. proposed irrigation area)

- Existing infrastructure

- Include all diagrams of the scheme from the source to the end use(s) in Appendix B.

Table 1 Summary of the Name-F11 recycling scheme

| Scheme Characteristic | Description |
| --- | --- |
| Location |  |
| Source of recycled water |  |
| Volumes of recycled water to be produced |  |
| Proposed end uses of the recycled water |  |
| Percentage of recycled water used in each one of the proposed end uses |  |
| Type of treatment system | (e.g. SBR Sequence Bach Reactor) |
| Location of the WWTP |  |
| Expected flows per day | Minimum Average Maximum |
| Peak inflow of the plant |  |
| Estimated number of people that will benefit/use the recycled water scheme |  |
| Irrigation area (m2) |  |
| Risk exposure level |  |

# 2. Recycled Water Quality Policy

## 2.1 Policy document

Develop a recycled water policy statement, endorsed by senior managers and implemented by the scheme including participant stakeholders. The policy needs to address the commitments of the organisation/scheme manager with the recycling scheme including commitment to high levels of management and monitoring through the life of the scheme as well as allocation of financial and human resources.

## 2.2 Regulatory and formal requirements

Identify and document all regulatory requirements of the scheme and list of relevant agencies.

This management plan will be submitted to DOHWA for review, as part of the conditions of approval for the Name-F11 recycling scheme.

## 2.3 Stakeholder engagement

Identify all stakeholders and their specific responsibilities, including the supplier, scheme manager and users affecting or affected by, decisions or activities of the scheme.

# 3. Roles and Responsibilities

Provide details of the responsibilities of each stakeholder involved in the scheme

## 3.1. Supplier

The supplier is the producer of the recycled water (in many cases is the Water Corporation)

Include the planning and implementation of activities conducted by the supplier to ensure the production of recycled water to a quality that is fit for purpose.

Refer to Appendix M: Recycled Water Supply Agreement or the MoU between the supplier and the scheme if applicable.

## 3.2 Scheme manager

The scheme manager is the body responsible for the management of the recycled water delivery system. In some schemes the scheme manager is also the responsible for the supply of recycled water.

Include the planning and implementation of activities conducted by the scheme manager to ensure the safe delivery of recycled water. Include information about the financial soundness of the scheme and a explicit statement about public liability insurance.

## 3.3 Users

The user is the person, organisation or community group that uses the recycled water

Include the planning and implementation of activities conducted by the scheme manager to ensure end users are informed, educated and trained of the “conditions of use” to use the recycled water in a safe manner.

## 3.4 Duty of care holder

Provide the contact details for the personnel whom have the duty of care to provide safe recycled water for the intended purposes.

The Duty of Care Holder for the Recycled Water Scheme is:

Name of Company/organisation

Address

The contact person for the Name-F11 Recycled Water Scheme is

Name of person:		___________________________

Position:			___________________________

Phone number on (08):	___________________________

## 3.5 List of contact details

| Name | Position | Phone |
| --- | --- | --- |

Refer to Appendix L: Incident Management Plan

# 4. Assessment of the Recycled Water Scheme

## 4.1 Source water (influent)

- Identify source of water

- Provide a description of the catchment, describing any system inputs (domestic, industrial or commercial discharges). A Catchment characterisation may be required for certain projects.

- Provide information on volumes and discharge patters into sewers if available (diurnal, seasonal)

- Provide data on the source water quality if available

- If no specific data available, provide information on the indicative number of microorganisms/contaminants in the source water from the literature

Table 2 Expected/measure water quality parameters of the source water (influent)

| Parameter | Units | Value |
| --- | --- | --- |
| E Coli | cfu/100 mL |  |
| Coliphage | pfu/100 mL |  |
| Clostridia | cfu/100 mL |  |
| BOD | mg/L |  |
| SS | mg/L |  |
| pH | pH units |  |
| Turbidity | NTU |  |
| TN | mg/L |  |
| TP | mg/L |  |

## 4.2 Water quality objectives

Chemicals are not generally considered to be a human health risk where wastewater is derived predominantly from domestic catchments, and recycled water is used for non-drinking purposes.

Industrial recycling schemes or other schemes in which chemical inputs from the catchments are considered to be significant, require the identification of specific chemical water quality objectives through risk assessment.

Describe the expected recycled water quality after the treatment.

Include indicative log reductions from treatment

Table 3 Water quality objectives of the recycled water

| Parameter | Units | Value |
| --- | --- | --- |
| E Coli | cfu/100 mL |  |
| Coliphage | pfu/100 mL |  |
| Clostridia | cfu/100 mL |  |
| BOD | mg/L |  |
| SS | mg/L |  |
| pH | pH units |  |
| Turbidity | NTU |  |
| TN | mg/L |  |
| TN | mg/L |  |

## 4.3 Health risk assessment

Health risk assessment is used to identify the most serious threats of recycled water based on likelihood of occurrence and severity of consequences. Health risk assessment evaluates the hazards and events that can compromise recycled water quality and safety.

Include a summary of

- Hazards

- Risks identified (including risks associated with unintended uses e.g. cross connections)

- Management priority (High, Medium, Low)

- Existing preventative measures

- Additional controls required to minimise risk

Detailed HRA is presented in Appendix E: Health Risk Assessment.

## 4.4 Recycled water storage

Include:

- Details and capacity of storage tank

- Security (accessibility to public)

- Turnover time

- Properly sealed? (accessibility to animals/insects)

- Maintenance

- Structure soundness (Australian Standards compliance)

- Refer to Appendix B: Drawings of the Recycled Water Scheme from Source to End-uses; Appendix D: Treatment Plant Process Flow Diagram, and/or Appendix N: Operational Monitoring and Critical Control Points

## 4.4. Reticulation network (distribution system) and application (receiving environment(s)

Map out the zones of the recycled water system.

- Refer to Appendix D: Treatment Plant Process Flow Diagram, and/or Appendix; Appendix V Environmental Management Plan and/or Appendix P: Operational and Maintenance Manual of the Irrigation System.

## 4.5 Systems operation

Describe briefly the systems operation from source to distribution.

## 4.6 Prevention of cross connection with potable supplies

Describe measures in place to minimise risk of cross-connection of the recycling scheme with potable water

Refer to Appendix E: Health Risk Assessment, Appendix L: Incident Management Plan and/or Appendix N: Operational Monitoring and Critical Control Points

# 5. Treatment Train

## 5.1 Treatment description

- Provide details of the treatment steps that will be used to treat the source water.

- Discuss the validation of treatment efficacy to remove contaminants from source water.

- Include details of alternative treatment systems that will be used in an event of treatment system failure.

Refer to Appendix D: Treatment Plant Process Flow Diagrams

## 5.2 The HACCP

A preventive risk management system such as Hazard Analysis and Critical Control Point (HACCP) shall be used for assessing risks and managing risk. The purpose of the HACCP is to identify all hazards in the treatment process that could cause the final product to be out of specification, such that it could cause illness or injury.

Refer to Appendix N: Operational Monitoring and Critical Control Points

## 5.3 Operational monitoring and maintenance

Include details on the operational monitoring and maintenance of all the major apparatus within the recycled water system including:

- Availability of operational monitoring and maintenance manual / procedures / checklists;

- Personnel in-charge of operational monitoring and maintenance; and

- Operational monitoring and maintenance frequencies.

- Refer to Appendix N: Operational Monitoring and Critical Control Points

## 5.4 System operators competency

- Provide details on the competency of personnel that will operate the system

## 5.5 Alarms, critical control points and critical limits

Include a summary of:

Identified critical control points (CCPs)

Critical limits and corrective actions

Describe the modes of plant operation

- Automatic

- Manual

- Star up and shut down procedures

The operation of shutdown systems must be fully tested at commissioning and the outcome of these tests recorded.

Real-time monitoring linked to an appropriate alarm monitoring system and automatic shut-down is required for all critical control points and must be available at all times.

List all measures in place to minimise the risk of cross-connection with drinking water

Table 4 Alarms message, effect on plant and corrective actions

| Alarm message | Affected components | Effect on plant | Corrective actions |
| --- | --- | --- | --- |
| Low Effluent Chlorine* | Sampling pump, pH/Cl analyser, NaOCl dosing | Effluent is out of specification, reuse will be suspended until cause is identified, rectified and alarm is reset | Check NaOCl tank level, verify dosing pump is running, confirm analyser is reading correctly |
| CCP1 |  |  |  |
| CCP2 |  |  |  |

Low Effluent Chlorine* provided as an example only

- Refer to Appendix N: Operational Monitoring and Critical Control Points and Appendix O: Operational and Maintenance Manual of the WWTP

## 5.6 Water supply arrangements

If a recycled water provider supplies recycled water to another recycled water provider, the description of infrastructure/delivery scheme can cease at the point where the water enters the other provider’s infrastructure.

  - Details about the quality of water being provided, or to be provided.

  - Details on the volume of recycled water being supplied for each of the uses per year.

- Include contract/agreement in Appendix M: Recycled Water Supply Agreement if applicable

# 6. Monitoring Plan

Monitoring will be undertaken as a two phase approach based on the AGWR as follows:

## 6.1 Validation monitoring

Validation is a critical component of treatment process management because it ensures that the required water quality objectives will be achieved.

Refer to section 6.5 Table 6.1 “Verification and Validation Monitoring Requirements” of the Guidelines for the Use of Recycled Water in WA (2010) for the validation monitoring requirements based on risk exposure level.

Include pathogen removal validation data from the manufacturer for individual processes, within the treatment train in Appendix Q: Validation Report of Individual Treatment Components

Validation monitoring will be undertaken during commissioning of the treatment plant in accordance with the Guidelines for the Use of Recycled Water in WA (2010). Before the scheme is approved, a validation and verification report of the treatment system needs to be reviewed by DOHWA.

Refer to Appendix F: Validation and Verification Report.

## 6.2 Operational monitoring

Operational monitoring refers to the parameters monitored at each CCP to demonstrate the treatment plant is operating correctly. Each operational parameter has critical limits and alert limits that are established during the validation monitoring. Corrective actions that need to be taken if a critical limit is breached need to be included.

The procedures for operational monitoring shall include:

- What is being monitored

- How the monitoring is done

- When the monitoring is done

- Where the monitoring is done

- Who is responsible for ensuring it is completed

- The critical limits, alert levels an corrective actions

Detail information is included in Appendix N: Operational Monitoring and Critical Control Points

## 6.3 Verification monitoring

Verification monitoring assesses the overall performance of the system and compliance with the overall water quality objectives. It is independent of the routine operational monitoring of the system and it is used to confirm product water quality objectives.

Testing need to be conducted by a laboratory that is National Association of Testing Authorities (NATA) accredited laboratory.

Refer to section 6.5 Table 6.2 “Operational Monitoring Requirements” of the Guidelines for the Use of Recycled Water in WA (2010) for the operational monitoring requirements based on risk exposure level.

For details refer to Appendix F: Validation and Verification Report and Appendix H: Sampling Plan.

7. Employee Training & Public Education

## 7.1 Employee awareness and involvement

Employees including plant operators and contractors shall have a sound knowledge base from which to make effective operational decisions. This requires training in the methods and skills required to perform their tasks efficiently and competently. Employees need to be aware of the potential consequences of system failures, and of how their decisions can affect the safety of the scheme.

Provide details on the suitability of the operators’ and contractors’ qualifications, training and experience.

## 7.2 Employee training

Detail employee training needs, programs and frequency.

## 7.3 Training records

Training records need to include the following information:

- Names of attendees

- Signature of attendees

- Signature of trainer

Refer to Appendix J: Training and Education

## 7.4 Occupational health and safety procedures

Include:

### 7.4.1 Recycled water inductions and hand-over procedures

### 7.4.2 Management measures

- Chemical storage and handling

- Hygiene practices in the workplace

- PPE

Refer to Appendix G: OHS Procedures

## 7.5 Public safety

Provide copies of the information to be given to recycled water users to promote awareness of recycled water quality issues, allowable uses, responsibilities and the impacts of unauthorised uses.

### 7.5.1 Educational material

Refer to Appendix I: Warning Signs

### 7.5.2 Management measures

# 8 Incident and Emergency Response

The recycling scheme shall include all possible incidences that will affect the water quality or any aspect of the recycled scheme. Include details of:

- Incident giving rise to hazard;

- Possible causes;

- Consequences;

- Risk (Likelihood and consequence);

- Response action and Personnel in-charge;

- Reporting protocols; and

- Preventative measures.

- Who is responsible for reporting incidents to DOHWA

Incidents and accidents provides a useful occasion to monitor and review risk and treatments and to gain insight on how the risk management process can be improved. Questions to be answered include:

- Did we previously identify and analyse the risk involved?

- Did we identify the actual causes in risk identification?

- Did we rate and assess risks and controls correctly?

- Did the control operate as intended?

- Were the treatment plans effective? If not, where could improvements be made?

- How our risk management in general be improved?

- Who needs to know about this leanings and how should be disseminate?

- What do we need to do to ensure that failure events are not repeated but that successes are?

## 8.1 Communication

## 8.2 Incident and emergency response protocols.

Detail incident and emergency protocols specific to the production and supply of recycled water including response actions, roles and responsibilities and communication arrangements.

Detail the arrangements and procedures for notification of incidents

The following must be immediately notified to DOHWA, Water Unit (Telephone 9388 4999)

- A system failure that may potentially impact on the users of the recycled water.

- An emergency or incident that potentially places public health at risk.

- Any changes to the RWQMP or operation of the treatment process that may potentially impact achieving the required water quality objectives

Refer to Appendix L: Incident Management Plan

# 9 Community Involvement and Consultation

## 9.1 Community consultation

Describe any community consultation process conducted

## 9.2 Communication

Describe how responsibilities will be understood and communicated to all stakeholders

# 10 Documentation and Reporting

## 10.1 Management of documentation and records

Document control procedure shall be implemented to ensure that all copies of documents referenced in the RWQMP are current and controlled. Include who is responsible for keeping the records and preparing the annual report.

Detail how documents will be managed

Records shall be kept for:

- Treatment plant monitoring results and analyses

- Breaches of critical limits and corrective actions taken

- Verification monitoring

- Incidents and emergencies and corrective actions taken

- Inspection and maintenance activities relevant to water quality

- Training activities

- Compliance records

## 10.2 Reporting

### 10.2.1 Internal

#### 10.2.1.1 Incident report

All employees and contractors shall record the initial incident information including:

- Date, time, location and nature of the incident.

- Persons injured, equipment damaged or environment impacted.

- Nature of injury or damage and estimate of severity.

- Immediate corrective action being taken.

- Assistance required.

- Activity in progress at the time.

### 10.2.2 External

## 10.3 Annual report

The scheme needs to submit the annual report by 30 September each year to DOHWA. The report corresponds to the immediately preceding financial year and contains the detailed information in Appendix K: Annual Report.  The report should be publicly available on the web site and available free of charge to the scheme users.

# 11 Evaluation and Audit

- Detail the process for reviewing and updating the RWQMP

- Describe the audit process, scope and frequency specifically relating to auditing for compliance of the system with the RWQMP, DOHWA conditions of approval and the AGWR.

- Describe the method of report

## 11.1 Long-term evaluation of results

## 11.2 Audit

Audit results shall be made available to DOHWA on request.

### 11.2.1 External audit

### 11.2.2 Internal audit

# References

Australian and New Zealand Standard AS/NZS 3500:2003, Plumbing and Drainage

Australian and New Zealand Standard HB 436:2004 Risk Management Guidelines Companion to AS/NZS 4360:2004

Australian and New Zealand Standard AS/NZS ISO 31000:2009 Risk Management – Principles and Guidelines

Australian and New Zealand Standard AS/NZSISO 10005:2006 Quality management systems – Guidelines for quality plans

enHealth Council (2004) Environmental Health Risk Assessment: Guidelines forassessing human health risk from environmental hazards, Department of Health and Ageing and enHealth Council, Canberra

NRMMC, EPHC and AHMC (Natural Resource Management Ministerial Council, Environment Protection and Heritage Council and Australian Health Ministers’ Conference) (2006) Australian Guidelines for Water Recycling: Managing Health and Environmental Risks (Phase 1) Canberra.

NSW Guidelines for management of private recycled water schemes (2007) Department of Water and energy NSW Government

Queensland Government, Natural Resources and Water (2008) Recycled water management plan and validation guidelines.

SA Health Recycled water systems: information guide for applicants Government  of South Australia

Department of Health, Western Australia :  Health Risk Assessment in Western Australia, Government of Western Australia Health risk assessment

Department of Health, Western Australia: Guidelines for the use of Recycled Water in Western Australia: Guidelines for the non-potable uses of recycled water in Western Australia (health.wa.gov.au)

Department of Health, Western Australia (2010) Recycled water sampling technique - Factsheet - Government of Western Australia Recycled water sampling technique (health.wa.gov.au)

Department of Health, Western Australia (2010) Warning signs for non-potable water schemes - Factsheet - Government of Western Australia

Warning signs for non-potable water schemes (health.wa.gov.au)

Department of Health, Western Australia, Health Risk Assessment (Scoping) Guidelines, Government of Western Australia: Health risk assessment

Department of Health, Western Australia Mosquitoes and Cyclones; Managing the Risk of Mosquito Borne Disease Guidelines 2006 Government of Western Australia Mosquitoes, cyclones and flooding (health.wa.gov.au)

Department of Health, Western Australia (2007) Chironomid midge and mosquito risk assessment guide for constructed water 2007 Government of Western Australia: Chironomid-midge.pdf (health.wa.gov.au)

Glossary

| Term/Abbreviation | Definition/Description |
| --- | --- |
| Activated Sludge | An activated mass of micro-organisms capable of stabilising waste aerobically – a “biomass”. |
| AGWR (2006) | Australian Guidelines for Water Recycling: Managing Health and Environmental Risk (Phase 1) |
| Alert limit | Is the early warning that process is becoming unstable. May indicate the possibility of exceeding a regulatory requirement or a critical limit for a CCP |
| AS/NZS | Australian Standards/New Zealand Standards |
| BOD | Biological oxygen demand |
| CCP | Critical control point. CCPs are activities, procedures or processes where a control can be applied and that is essential for preventing, eliminating or managing a hazard to an acceptable level. |
| cfu | Coliform forming units |
| COD | Chemical oxygen demand |
| Commissioning verification | A validation monitoring conducted to the treated water to prove that the expected water quality is being constantly produced by the commissioned treatment plant |
| Critical limit | A prescribed tolerance that must be met to ensure that a critical control point effectively controls a potential health hazard; a criterion that separated acceptability from unacceptability. |
| Ct | The production of disinfectant concentration (C in mg/L) and contact time (t in minutes), used in disinfection |
| DO | Dissolved oxygen |
| E Coli | Escherichia coli. The most common thermotolerant coliform present in faeces and used as indicator of faecal contamination |
| Exceedance | Water quality that goes outside identified limits. This limits may include alert levels, critical limits or water quality objectives |
| g | gram |
| GL | gigalitre |
| HACCP | Hazard analysis and critical control points |
| Hazard | A biological, chemical, physical or radiological agent that has the potential to cause harm |
| Hazardous event | An incident or situation that can lead to the presence of a hazard, or often directly contains a hazard |
| kL | Kilolitre |
| L | Litre |
| Log reduction | Used in reference to the physical-chemical treatment of water to remove, kill or inactivate microorganisms such as bacteria, protozoa and viruses (1-log removal = 90% reduction in density of the target organism, 2-log removal = 99% reduction, 3-log removal = 99.9% reduction, etc). |
| MBR | Membrane bioreactor |
| mg/L | Milligrams per litre |
| ML | Megalitre |
| MoU | Memorandum of understanding |
| NATA | National Association of Testing Authorities |
| NTU | Nephelometric turbidity unit |
| Operational monitoring | The routine monitoring of control parameters that confirm the treatment process are under control and operating within the operating criteria |
| PCT | Process control table |
| PLC | Programmable logic controller |
| Point of supply | The physical point of transfer to the user |
| PPE | Personal protective equipment |
| Public open space | Any open space, such as parks, sporting fields, racecourses, school ovals, municipal parks and gardens, golf courses, footpaths, car parks, road verges, where either members of the public, staff or employees may be exposed to recycled water. It does not include gardens in domestic residences nor agricultural farmland |
| QA/QC | Quality assurance/quality control |
| Recycled water provider | An entity that owns infrastructure for the production and supply of recycled water |
| Residual risk | The risk remaining after consideration of existing preventive measures |
| Risk | The likelihood of a hazard causing harm in exposed populations in a specified time frame, including the magnitude of that harm. |
| RWQMP | Recycled water quality management plan |
| SBR | Sequential batch reactor |
| SCADA | Supervisory control and data acquisition |
| Scheme | Refers to a recycled water scheme |
| Sewage | Household and commercial wastewater that contains or may contain faecal, urinary or any other human waste |
| Sewerage | A sewer, access chamber, vent, engine, pump, structure, machinery, outfall or other work used to receive, store, transport or treat sewage |
| Sludge | The constituents removed from wastewater treatment plants include screenings, grit, scum and sludge. Sludge is usually in the form of a liquid or semisolid liquid that typically contains from 0.25 to 12% solids by weight. |
| SS | Suspended solids |
| Target limit | Refers to the normal operational value(or value range) for a process which indicates that the process is stable |
| TN | Total nitrogen |
| TP | Total phosphorous |
| Treatment Validation | A documented program about how the plant or equipment used for the treatment of recycled water are to be tested to demonstrate the quality f the recycled water consistency meets the water quality objectives for the intended end use. Treatment validation includes the operational criteria to ensure capability to effectively control hazards |
| TSS | Total suspended solids |
| UV | Ultraviolet |
| Violation limit | Critical limit is the operational value or performance criteria (that has been violated) which separates acceptable from unacceptable in terms of hazard control and recycled water safety. Corrective actions need to be implemented immediately to regain process control |
| WWTP | wastewater treatment plant |

# Appendices

Required for all Projects:

Appendix A: 	Scheme Location

Appendix B:	Drawing of the Recycled Water Scheme from Source to End-use(s)

Appendix C:	Water Balance

Appendix D:	Treatment Plant Process Flow Diagrams

Appendix E:  	Health Risk Assessment

Appendix F:	Validation and Verification Report

Appendix G:	OHS Procedures and Material Safety Data Sheets

Appendix H:	Sampling Program

Appendix I:	Warning Signs

Appendix J:	Training and Education

Appendix K:	Annual Report

Required on a case by case basis:

Appendix P:	Recycled Water Supply Agreement

Appendix Q:	Operational Monitoring and Critical Control Points

Appendix R:	Operational and Maintenance Manual of the WWTP

Appendix S:	Operational and Maintenance Manual of the Irrigation System

Appendix T:	Validation Report of Individual Treatment Train

Appendix U:  Incident Management Plan

Appendix V:	Mosquito Management Procedure

Appendix W:	Land Capability Assessment

Appendix X:	Environmental Assessment

Appendix Y:	Environmental Management Plan

Appendix Z:	Catchment Characterisation

# Appendix A: Scheme Location

Include

- Location map

- Site photographs

Appendix B: Drawing of the Recycled Water Scheme from Source to End-use(s)

Include a schematic overview of the recycling scheme with:

- Location of source water

- Location of treatment plat

- Storages pipeline route(s) and

- End uses

Appendix C: Water Balance

Include:

- Assumed scheme occupancy patter

- Volumes of wastewater produced by person and in total per day

- Recycled water demand (e.g. irrigation demands, maximum design irrigation rate)

- Climate data

- Stormwater

Table Scheme water balance in (ML)

|  | Jan | Feb | Mar | Apr | Jun | Jul | Aug | Sep | Oct | Nov | Dec | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scheme Inputs |  |  |  |  |  |  |  |  |  |  |  |  |
| Rainfall |  |  |  |  |  |  |  |  |  |  |  |  |
| Potable water supply |  |  |  |  |  |  |  |  |  |  |  |  |
| Groundwater abstraction |  |  |  |  |  |  |  |  |  |  |  |  |
| Other |  |  |  |  |  |  |  |  |  |  |  |  |
| Total inputs |  |  |  |  |  |  |  |  |  |  |  |  |
| Scheme Outputs |  |  |  |  |  |  |  |  |  |  |  |  |
| End-use 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| End use 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Stormwater system |  |  |  |  |  |  |  |  |  |  |  |  |
| Sewer System |  |  |  |  |  |  |  |  |  |  |  |  |
| Other |  |  |  |  |  |  |  |  |  |  |  |  |
| Total outputs |  |  |  |  |  |  |  |  |  |  |  |  |

Appendix D: Treatment Plant Process Flow Diagrams

Include:

- A site plan showing the general arrangement of the treatment facility

- A process flow diagram, and

- A process instrumentation diagram.

- Location of critical control points

- Capacity of tanks

# Appendix E: Health Risk Assessment

## 1. Hazard identification

Sources of potential hazards shall be identified

### 1.1 Microbial hazards

### 1.2 Chemical hazards

### 1.3 Hazardous events and other hazards

Potential hazardous events shall be identified and listed at each step in the process flow.

Include the unauthorised or incorrect use of the recycled water and the onsite controls that need to be implemented to minimise the risk.

## 2. Dose response

## 3. Exposure assessment

### 3.1 Routes of exposure

#### 3.1.1 Dermal

#### 3.1.2 Direct ingestion

#### 3.1.3 Inhalation

## 4. Risk characterisation

Risk characterisation integrates the information gathered in the hazard identification, dose response and exposure assessment.

When assessing the risk consider:

- The number of people exposed

- How often they are exposed

- How long they are exposed for

- Any combination of hazards

- The seriousness or severity of any consequence

Table 1 Qualitative measures of health consequences

| Category | Acute health consequences (per hazard or outbreak) | Chronic health effect requiring: |
| --- | --- | --- |
| Catastrophic 1 | Health: Major impact for a large population >1 fatality or Major impact for a large population OR >5 permanent disabilities OR Non-permanent injuries requiring hospitalisation for 5-10% of population at risk OR Acute health effect requiring hospitalisation for >5-10% of population at risk Environment: Potentially lethal to regional ecosystem or threatened species; widespread on-site and off-site impacts | Medical treatment for 10-15% of population at-risk* |
| Major 2 | Health: Major impact for a small population No fatality AND (1 permanent disability OR Non-permanent injuries requiring hospitalisation for >1-5% of population at risk OR Acute health effect requiring hospitalisation for >1-5% of population at risk OR Evacuation is necessary Environment: Potentially lethal to local ecosystem; predominantly local, but potential for off-site impacts. | Medical treatment for 2-10% of population at-risk* |
| Moderate 3 | Health: Minor impact for large population No fatality AND No permanent disability AND (Non-permanent injuries requiring hospitalisation for 1-2% of population at risk OR Acute health effect requiring hospitalisation for 1-2% of population at risk AND No evacuation Environment: Potentially harmful to regional ecosystem with local impacts primary contained to on-site. | Medical treatment for 1-2% of population at-risk* |
| Minor 4 | Health: Minor impact for small population No fatality AND No permanent disability AND (Non-permanent injuries requiring hospitalisation for 1-5 persons OR no acute health effect requiring hospitalisation) AND No evacuation Environment: Potentially harmful to local ecosystem with local impacts contained to on-site. | Medical treatment for about 0-1% of population at-risk* |
| Insignificant 5 | Insignificant impact or not detectable | No chronic health effect |

*Estimated average size of population at-risk across project lifecycle

Table 2 Qualitative measures of likelihood

| Level | Likelihood descriptor | Frequency of incident or outbreak with non-chronic health effect | % Chance of chronic health effect during life of project |
| --- | --- | --- | --- |
| 1 | Rare/remote | Once in more than 100 years May happen only in exceptional circumstances | Up to 5% |
| 2 | Unlikely | Could occur within 20 years May happen in unusual circumstances | 6 - 30% |
| 3 | Possible/ occasionally | Once in 5 – 10 years May happen | 31% - 60% |
| 4 | Likely | Once in 1 to 5 years May easily happen | 61% - 90% |
| 5 | Almost certain | More than once a year Expected to happen many times within a year | Over 90% |

Table 3 Qualitative risk estimation

| Likelihood | Consequences |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Insignificant | Minor | Moderate | Major | Catastrophic |
| Almost Certain | Low | Moderate | High | Very High | Very High |
| Likely | Low | Moderate | High | Very High | Very High |
| Possible | Low | Moderate | High | Very High | Very High |
| Unlikely | Low | Low | Moderate | High | Very High |
| Rare/remote | Low | Low | Low | High | High |

## 5. Risk mitigation

The final output from Appendix E (Health Risk Assessment) will produce a ranked table with all hazards and hazardous events. Risk mitigation table include the control measures that need to be implemented to minimise risk to an acceptable level.

Complete the table based on the health risk assessment

Include actions related to wastewater catchment management

Table 4 Risk characterisation and mitigation

| Hazard | Likelihood | Consequence | Qualitative risk | Mitigation (existing/proposed controls) |
| --- | --- | --- | --- | --- |
| * Mosquito breeding in storage tanks (e.g.) | Unlikely | Major | High | - Ensure sealed lids - Chemical treatment of active mosquito populations - Education on Mosquito Borne diseases - Mosquito management manual |

*Mosquito breeding given as an example

# Appendix F: Validation and Verification Report

## 1. Introduction

This report documents the testing of the microbiological removal and the effluent water quality of the Name-F11 recycling scheme.

The treatment plant comprises the following processes:

List

  - 1

  - 2

  - 3

  - 4

## 2. Sampling methodology

Include:

- The parameters tested

- The frequency of testing

- The sampling location(s)

- Quality assurance/Quality control (QA/QC)

## 3. Results

Include:

- A summary of test results for the commissioning verification

- A summary of any water quality exceedances (if they occur), the circumstances that led to them occurring and any corrective/preventive actions taken or put in place

- A statistical analysis of the data collected to determine the confidence level for meeting the water quality objectives on a continuous basis.

Table 1 Summary of test results

| Date | Report No | Lab | Parameter | Units | Effluent Compliance value | Sampling Point | Result | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

## 4. Operational/online data

Table 2 Summary of operational data results

| Date | Parameter | Units | Sampling Point | Result (Daily average) |
| --- | --- | --- | --- | --- |

## 5. Influent flow

## 6. Discussion

## 7. Conclusions

## 8. Copy of original results

# Appendix G: OHS Procedures and Material Safety Data Sheets

Include material safety data sheets of all chemicals used in the treatment plant and the end uses (e.g. irrigation system)

# Appendix H: Sampling Plan

## 1. Safety procedures

## 2. General rules of sampling

## 3. Sample bottle labelling

## 4. Microbiological collection technique

- Equipment required

- Preparation

- Procedure

## 5. Chemical collection technique

- Equipment required

- Preparation

- Procedure

## 6. Chlorine residual measurement

- Equipment required

- Preparation

- Procedure

## 7. Sample preservation and transportation

## 8. Documentation/records

## 9. Site code number(s) and their map location

## 10. List of laboratories

| Laboratory Name |  |
| --- | --- |
| Contact Person: |  |
| Position: |  |
| Business Address: |  |
| Phone: |  |
| Fax: |  |
| E-Mail: |  |
| Laboratory Name |  |
| Contact Person: |  |
| Position: |  |
| Business Address: |  |
| Phone: |  |
| Fax: |  |
| E-Mail: |  |

Include the Sampling Technique Factsheet from DOHWA.

Appendix I: Warning Signs

Include photos of the scheme showing the location of the scheme warning signs

Include the Warning Signs Factsheet from DOHWA

Appendix J: Training/ Education

## 1. Operator training manual

Include training manual for WWTP operators

## 2. Induction program and hand-over procedures

## 3. Training of sub-contractors

## 4. Training forms

e.g. Training declaration form, list of training attendance form

## 5. Educational material

# Appendix K: Annual Report

Recycled Water Annual Report 2022/2023

- Introduction

Table 1: Overview of the recycled water scheme

| Summary of the recycling scheme |  |
| --- | --- |
| Name of Scheme |  |
| Recipient |  |
| Approval Number |  |
| Location of the Scheme |  |
| Risk level |  |
| Contact details |  |
| Name |  |
| Position/Organisation |  |
| Street/Postal Address |  |
| Tel No |  |
| Email |  |

- Scheme Operational months

Please select the months that scheme is active

06/12/2022	To              06/12/2022

- Scheme Volume

| Recycled Water Approval Threshold |  |
| --- | --- |
| *Recycled Water Volume Used |  |
| Comments (if any) |  |

*Only report the Recycled water volumes not combined volumes such as recycled water and stormwater/bore water

- Quality of recycled water

The utilization of recycled water must adhere to the approved conditions in order to maintain its quality. If the scheme requires monitoring of chlorine levels, UV, pH, turbidity, and SS, these parameters should be recorded and kept in records. During the audit process, these records will be assessed to ensure compliance with the approved conditions.

Include:

- Site code number of recycled water quality sampling

- A tabulated presentation of laboratory recycled water quality test results.

- An assessment of recycled water quality compliance with the Guidelines for the use of Recycled Water in WA values

| Location (Site code) | Parameter | Units | Frequency of sampling (e.g weekly, monthly) | Total No of samples | Max sampling value | Total number of samples within approval condition | Total number not complying |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Table 2: Microbial Summary of laboratory results: include monitoring results for E.coli, clostridia and coliphages (Monitoring of clostridia and coliphages are required for some high risk recycled water schemes) and Helminths (for Kimberley region, north of the 20th parallel recycled water scheme) . Please refer to the condition of approval for microbial monitoring requirements.

- Table 3: Physio-chemical parameters of recycled water: Physio-Chlorine levels, pH, Turbidity and SS at the time of microbial sampling. Please refer to the condition of approval for monitoring requirements.

| Location (Site code) | Parameter | Units | Frequency of sampling (e.g weekly, monthly) | Total No of samples | Max sampling value | Total number of samples within approval condition | Total number not complying |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Graph 1 (Optional): Online monitoring of Physio-chemical data: If applicable, provide a graph of online monitoring of physio-chemical data which is to be collected through online or daily data , and lab certificates. E.g:

.

- 3.1 Sampling Summary/exceedances

- Include explanation for:

  - Exceedances

  - Missing samples

  - Resampling results

- Emergency and Incident Management

A summary of events and issues that affected recycled water quality and/or caused a non-compliance with the guideline values. Please refer to the emergency response protocol page 42 of the Guidelines for the non-potable uses of recycled water in Western Australia (health.wa.gov.au)) E.g.:

- recycled water scheme where two consecutive microbial results at regulatory sampling point(s) are above the health-based water quality objective as defined in the DoH approval

- algal bloom exceeds

- Cessation or incident within the recycling supply scheme for events with an anticipated duration of greater than 24 hours

Sewage spill

- Table 4: Incident summary

| Incident | Details of the incident | Actions to investigate/correct | Notification to DoH (if applicable) |
| --- | --- | --- | --- |

- Major Infrastructure Works Carried Out

Have any of the following changes occurred to the recycled water scheme? Yes/No

- Major changes to WWTP

- Change to disinfection system where different from WWTP

- Existing scheme – change to storage system (new or different size storage tank/pond not a like for like replacement)

- Existing scheme – new end use location

- Sampling point location change

- Complaints

Include a summary of public/staff complaints regarding recycled water.

Table 5: Summary of complaints relating to recycled water Improvement plan

| Type/nature of the complaint | Location | Actions to investigate/correct | Notification to DoH (if applicable) |
| --- | --- | --- | --- |
| (e.g. odour/suspected illness/ ponding) |  |  |  |

- Audit summary

Table 6: Audit Summary: An internal audit was completed on the (Date), action items and responses outlined in Table below.

| Auditor/ reviewer comment (System deficiency and non-compliances) | Scheme response | Corrective actions to prevent reoccurrence | Timetable for corrective/preventive action | Person(s) responsible |
| --- | --- | --- | --- | --- |

- Documentation updates

| Documents | Date of last update/ Expiry date |
| --- | --- |
| Recycled Water Management Plan |  |
| Operational Maintenance manual |  |
| Recycled Water supplier agreement/ Memorandum of Understanding |  |
| Other |  |

- Other Comments

E.g. Request for updated approval, changes to the sampling locations, addition of new locations etc.

- Signature of the scheme manager

- ___________________

Reports to be signed off by the scheme manager.

# Appendix L: Incident Management Plan

Notification procedures and adequate contingency provisions to address unforeseen events which may lead to health or environmental harm or nuisance, including

Wastewater/ recycled water overflow

Failure or breakdown of wastewater pumps, pipes or equipment

- Power failure or interruption

- Natural events such as floods and fires, cyclones or heavy storms

- Discharge of hazardous substances to the wastewater stream

- Malicious actions and vandalism

The purpose of this document is to overview the incident management process for the Name-F11 recycling scheme. The document outlines the process of managing incidents as part of compliance management.

## 1. Incident classification

Minor incident/ significant incident/ major incident

## 2. Notification process

Include how notification of incidents will be made

This procedure is most effectively presented using a flow diagram and should include:

- The scheme’s internal protocol for reporting an incident to responsible staff (including main and back-up staff names, titles and phone numbers)

- the protocol (including phone numbers) for reporting to relevant emergency authorities, eg Department of Environment and Conservation, Swan River Trust, Fire and Emergency Service (FESA), Department of Health (DH), local government, etc.

## 3. Emergency communications contact list

Environmental incidents must be reported using the Emergency Pollution Response line 1300 784 782 or to EPA: 1800 100 833.

Incident reporting sheets need to include the following details:

• location of the incident

• time and date of the incident

• nature of the release (estimated quantity, quantity and receiving environment)

• licensee’s name if applicable

• name and telephone number of contact person.

| Name: |  |
| --- | --- |
| Position: |  |
| Business Address: |  |
| Phone: |  |
| Mobile Phone: |  |
| Fax: |  |
| E-Mail: |  |
| Name: |  |
| Position: |  |
| Business Address: |  |
| Phone: |  |
| Mobile Phone: |  |
| Fax: |  |
| E-Mail: |  |
| Name: |  |
| Position: |  |
| Business Address: |  |
| Phone: |  |
| Mobile Phone: |  |
| Fax: |  |
| E-Mail: |  |

## 4. Containment and remediation procedures

## 5. Incident cause and investigation procedures

# Appendix M: Recycled Water Supply Agreement

Include the agreement, contract or MoU between the Recycled Water Supplier and the Water Recycling Scheme Manager

The document shall include:

  - Details about the quality of water being provided, or to be provided.

  - Details on the volume of recycled water being supplied for each of the uses per year.

  - The commitment for continuous supply for essential services and contingencies for supply

Appendix N: Operational Monitoring and Critical Control Points

Operational monitoring is the routine, often continuous (on-line), monitoring of Critical Control Points and Process Control Points in the WWTP to ensure compliance within the defined operational criteria. It is used to determine if each preventative measure is effectively controlling hazards, and provides an advance warning if treatment barriers are moving away from a stable operational state.

CCP require operational parameters that can be measured and for which critical limits can be set to define the operational effectiveness of the activity. The operational parameters need to be monitored sufficiently frequently to reveal any failures in a timely manner. CCP also have process for corrective actions that can be implemented in response to deviations from critical limits and alert levels. Alert levels are more conservative than critical limits and represent an early warning so that corrective actions can be implemented before a critical limit is exceeded.

- Identify operational procedures required for all key processes and activities applying to the recycled water system.

- Describe how operational performance of the recycled water system will be ensured.

- Provide specific reference to CCPs and their mechanisms for operational control.

- Include statements on the capability and reliability of this equipment

- Provide copies of preliminary operating procedures relating to all key processes and activities (including all CCPs) in a preliminary operations manual. These must include the critical limits, corrective actions and detail when the system can be brought back online after a shut-down.

- Describe use of alarm systems where applicable.

- Provide links to monitoring protocols, including sampling, testing, auditing and equipment calibration.

- Provide a table listing corrective actions for non-compliance of all key operational parameters (specifically CCPs), e.g. disposal of non-compliant water if key processes fail.

Table P.1 Operational Monitoring Parameters

| Critical Control Point | Hazard removed by CCP | Parameter monitored | Rationale | Monitoring Type/ Frequency | Target range | Target Limit | Critical Limit (Violation) | Corrective Actions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Membrane filtration* | Particulates, pathogens | Turbidity | Monitoring the MF filtrate turbidity confirms that the membrane barrier has not had a significant breach detectable by a turbidity increase that would be due to solids ‘breaking through’ | Continual on-line monitoring | < 0.15 NTU | < 0.15 NTU | < 0.3 NTU | Alert: initiate backwash Violation: Take unit out of service |

*Membrane filtration given as an example

# Appendix O: Operational and Maintenance Manual of the WWTP

## 1. Overview of treatment process

## 2. System Drawings

- Process schematic

- Site plans

- Electrical drawings

## 3. Plant Sart-up

- Pre start-up checks

- Initial Start up

## 4. Alarm systems

## 5. Sludge Management

## 6. Equipment

- Quality certification

- Plant and equipment technical information

- Electrical and PLC equipment technical information

- Equipment Failure

- Equipment suppliers schedule (Include list of recommended spare parts that should be held on-site to enable timely rectification of any faults)

## 7. Materials and chemicals

## 8. Troubleshooting Guide

Table 1 Operational troubleshooting

| Problem | Possible Cause | Possible Solution |
| --- | --- | --- |

## 9. Plant safety and personal hygiene

## 10. Plant monitoring

- Routine inspection, monitoring and maintenance

## 11. Sample collection

## 12. Maintenance

- General plant maintenance

- Overall maintenance schedule timetable

- Include daily/weekly/monthly/three monthly inspection reports

## 13 Communications protocol for emergency and contingency situations

Table 2 Contact details for contingency situations

| Contact Person | Company | Position | Mobile phone |
| --- | --- | --- | --- |

# Appendix P: Operational and Maintenance Manual of the Irrigation System

## 1. Map of the pipeline route from WWTP to irrigation sites

## 2. Irrigation map

## 3. Areas of responsibility

Include responsibilities/tasks for all staff involved in the operations and maintenance of the irrigation system (e.g. watering, application of fertilisers)

4. Maintenance of storage tank, reticulation system

- General plant maintenance

- Overall maintenance schedule timetable

- Include daily/weekly/monthly/three monthly inspection reports

- Include specific forms used (e.g. Pump station audit sheet,  distribution system audit sheet)

## 5. Irrigation compliance

| Location | Day (start) | Time on | Time off | Comment |
| --- | --- | --- | --- | --- |

## 6. Warning Protocol

Colour pipe and warning signs

## 7. Contingency/emergency plan

| Incident | Responsible person/s | Action to be taken |
| --- | --- | --- |

# Appendix Q: Validation Report of Individual Treatment Components

Pre-commissioning validation is undertaken as part of the feasibility study, to determine what treatment process will be required to meet the water quality objectives.

Manufacturers specifications can be used to support the validation process but any removal rates provided by the manufacturer should be confirmed by monitoring during commissioning of the plant.

Include the following information for each one of the treatment components (e.g. ultrafiltration)

## 1. System description

## 2. Validation ranges

Include methodologies and conditions under which experiments were performed

3. Results

Include log reductions achieved by the treatment component

# Appendix R: Mosquito Management Plan

## 1. Introduction

Include purpose and definitions

Identify sources and extent of mosquito impacts based on:

- Previous mosquito surveys or reports if they exist

- Public complaints (most local governments keep a complaints register)

- Geographical survey:

- location of man-made water infrastructure  (e.g. sewage lagoons, constructed wetlands, rainwater and effluent re-use tanks, roadside drains and culverts)

- maps, aerial photographs

- local knowledge

- Land ownership & responsibilities (council planners, Dep’t of Land Administration)

- Applicable environmental legislation (council planners; environmental agencies)

## 2. Mosquito breeding habitats and human risk

Include potential mosquito breeding locations and periods of the year with increased risk (e.g. cyclone season)

If there is no prior information about mosquito breeding sites, seasonal productivity and the most prevalent

species, then the following baseline surveys will be essential.

- Larval surveys: survey all potential mosquito breeding habitats, natural and man-made

- Adult surveys: undertake adult mosquito trapping in a range of natural and domestic locations

- Timing of surveys: surveys should follow breeding triggers [e.g. rainfall, irrigation, dam releases, effluent re-use)] to maximise the effectiveness of the survey to locate breeding sites

- Prioritise surveys in areas closest to residential and recreational areas and work out from there

## 3. Risk management measures

List procedures/measures in place to minimise risk of exposure

The analysis of existing information and baseline mosquito surveys (above) will allow you decide whether,

when, where and how mosquito management should be undertaken.

## 4. Roles and responsibilities

The end-users of the recycled water have a key role and responsibility in any integrated program to manage mosquitoes. It is important to that users are kept informed and become stakeholders in achieving a successful mosquito management procedure.

# Appendix S: Algae Management Plan

High concentration of nutrient in recycled water can lead to algae blooms in water storages exposed to the sun. Similarly long periods of dry stable warm weather provide favourable conditions for algae blooms. Recycled water storages (reservoirs, dams and tanks) could also suffer from excessive algal growth and eutrophication. In addition to producing toxins, algae in recycled waters may clog irrigation systems.

## 1. Introduction

Identify sources and extent of algae bloom impacts based on:

- Local knowledge

- Previous data or reports if they exist

- Location of man-made water infrastructure  (e.g. sewage lagoons, constructed wetlands, rainwater and effluent re-use tanks, roadside drains and culverts)

- Maps, aerial photographs

## 2. Risk management measures to prevent algae blooms

List procedures/measures in place to minimise risk algae bloom formation including:

- Source control or catchment management practices to reduce nutrient inputs

- Treatment barriers used to reduce nutrient concentrations in the plant effluent

- Restriction of light entering storage systems

- Measures to control storage times  and stagnation prevention

- Visual daily monitoring during high risk periods

- Turbidity management

## 3. Risk management practices in algae blooms

Algae blooms should be treated as toxic until species have been identified and are considered safe.

List procedures/measures in place to manage algae bloom events including:

- Monitoring of algae levels

- Limitation of light entering storage systems

- Chemical treatment

- Physical controls

## 4. Incident Response

Include contact details of people responsible.

# Appendix T: Land Capability Assessment

## 1. Site Characteristics

## 2. Irrigation assessment requirements

## 3. Irrigation areas

## 4. Soils

## 5. Irrigation application rates (L/m2)

Include geotechnical report if available

# Appendix U: Environmental Risk Assessment

## 1. Routes of Exposure and environmental end-points

### Include:

### Exposure pathways

### Receiving environment

### Environmental end-points (soil, surface water, biota..)

## 2. Environmental risk assessment

## 3. Risk mitigation

# Appendix V: Environmental Management Plan

## 1. Soils description

Include:

- Soil type

- Phosphorus retention index

- Acid sulphate soils

- Imported soils

The key physical properties that should be identified in initial soil investigations include:

•	Texture/structure

• 	Topsoil depth

• 	Depth to drainage or root impeding layers

• 	Infiltration rates

• 	Soil−water holding capacities

## 2. Water resources

Include:

- Groundwater table

- Distance to surface waters

## 3. Land management

## 4. Nutrient and irrigation management plan

## 5. Proposed irrigation scheme

## 6. Nutrient application

## 7. Drainage management

## 8. Vegetation in the irrigated area

## 9. Pesticide storage and use

## 10. Sample locations (soil, groundwater, surface water)

# Appendix W: Odour Management Plan

## Odour assessment

Include Guidelines, software, modelling or data used

## Odour control measures

Include control measures to be implemented to limit odour releases for primary, secondary treatment and sludge handling. Include measures such as covers, odour treatment units, stacks and buffer distances.

## Odour monitoring

Inlclude periodic monitoring during operations

- Complaint registration and response

- Odour surveys

## Standard operational procedures

Include operational activities such as checks of flows and loading or replacement of scrubber media.

## Contingency plans

Inlclude contingency plans and complaint procedures for

- Upsets or maintenance

- In the event of exceedances

# Appendix X: Noise Management Plan

## 1. Noise assessment

Include background noise levels, noise modelling and noise prediction

## 2. Noise management

Include control measures to be implemented to limit noise

## 3. Noise monitoring

Inlclude periodic monitoring during operations and complaint registration and response

## 4. Standard operational procedures

Include operational activities to minimise/control noise.

## 5. Contingency plans

Inlclude contingency plans and complaint procedures for

- Upsets or maintenance

- In the event of exceedances

# Appendix Y: Catchment Characterisation

Include:

- Commercial inputs

- Industrial inputs

- Residential inputs

- Land uses

# Appendix Z: Audit Report

## 1. Introduction

## 2. Audit Methodology

## 3. Findings

## 4. Discussion

## 5. Recommendations